代码 1：七路循迹信号采集与偏差计算功能实现

static const int16_t line_weights[LINE_TRACKER_CHANNELS] =
{
  50,
  33,
  16,
  0,
  -16,
  -33,
  -50,
};

void LineTracker_Init(void)
{
  LineTracker_Update();
}

void LineTracker_Update(void)
{
  uint8_t raw_mask = 0U;
  uint8_t active_mask = 0U;
  uint8_t active_count = 0U;
  int16_t error = 0;
  int32_t weighted_sum = 0;
  uint32_t index = 0U;

  for (index = 0U; index < LINE_TRACKER_CHANNELS; index++)
  {
    uint8_t is_set =
        (DL_GPIO_readPins(line_ports[index],
                          line_pins[index]) != 0U) ?
        1U : 0U;
    uint8_t is_active = 0U;

    if (is_set != 0U)
    {
      raw_mask |= (uint8_t)(1U << index);
    }

    is_active = (g_line.active_low != 0U) ?
        (uint8_t)(is_set == 0U) : is_set;
    if (is_active != 0U)
    {
      active_mask |= (uint8_t)(1U << index);
      weighted_sum += line_weights[index];
      active_count++;
    }
  }

  g_line.raw_mask = raw_mask;
  g_line.active_mask = active_mask;
  g_line.active_count = active_count;
  g_line.line_seen = (active_count > 0U) ? 1U : 0U;

  if (active_count > 0U)
  {
    error = (int16_t)(weighted_sum / (int32_t)active_count);

    g_line.right_angle_detected = 0U;
    g_line.right_angle_direction = 0;

    if (g_line.right_angle_enable != 0U)
    {
      uint8_t left_right_angle =
          ((active_mask & 0x0FU) == 0x0FU) ? 1U : 0U;
      uint8_t right_right_angle =
          ((active_mask & 0x78U) == 0x78U) ? 1U : 0U;

      if ((left_right_angle != 0U) && (right_right_angle == 0U))
      {
        error = g_line.right_angle_error;
        g_line.right_angle_detected = 1U;
        g_line.right_angle_direction = 1;
      }
      else if ((right_right_angle != 0U) &&
               (left_right_angle == 0U))
      {
        error = (int16_t)-g_line.right_angle_error;
        g_line.right_angle_detected = 1U;
        g_line.right_angle_direction = -1;
      }
    }

    g_line.position = error;
    g_line.error = error;
  }
  else
  {
    g_line.right_angle_detected = 0U;
    g_line.right_angle_direction = 0;
  }

  g_line.sample_tick++;
}

说明：该代码采用七通道位置权值对有效传感器进行加权平均，得到车辆相对赛道中心的横向偏差；同时通过位掩码识别左右直角特征，为后续转向控制提供方向和偏差输入。

代码 2：电机速度 PID 与循迹 PID 功能实现

static int16_t Car_PidStep(volatile CarMotor_t *motor)
{
  float error = (float)motor->target_counts -
                (float)motor->measured_counts;
  float derivative = error - motor->pid.previous_error;
  float output = 0.0f;

  motor->pid.integral += error;
  motor->pid.integral = Car_LimitFloat(
      motor->pid.integral, motor->pid.integral_limit);

  output = (motor->pid.kp * error) +
           (motor->pid.ki * motor->pid.integral) +
           (motor->pid.kd * derivative);

  motor->pid.previous_error = error;
  output = Car_LimitFloat(output, motor->pid.output_limit);

  return Car_LimitPwm((int32_t)output);
}

static int32_t Car_LinePidStep(int32_t error_counts)
{
  volatile CarPid_t *pid = &g_car.line.pid;
  float error = (float)error_counts;
  float derivative = error - pid->previous_error;
  float output = 0.0f;

  pid->integral += error;
  pid->integral = Car_LimitFloat(
      pid->integral, pid->integral_limit);

  output = (pid->kp * error) +
           (pid->ki * pid->integral) +
           (pid->kd * derivative) -
           (g_car.line.gyro_damping * g_car.line.gyro_z);

  pid->previous_error = error;
  output = Car_LimitFloat(output, pid->output_limit);

  return (int32_t)output;
}

说明：电机速度环根据目标编码器计数与实测计数形成闭环调速；循迹环在比例、积分和微分项基础上引入陀螺仪阻尼，用于抑制车辆偏航振荡。两级输出均进行积分与输出限幅，以降低积分饱和和执行器过驱动风险。

代码 3：比赛运行平滑加减速规划功能实现

static void CompetitionTasks_UpdateSpeedProfile(uint32_t tick)
{
  uint32_t motion_ticks = tick - s_motion_start_tick;
  uint32_t command = 0U;

  if (motion_ticks < COMPETITION_ACCEL_TICKS)
  {
      uint32_t blend = CompetitionTasks_SmoothStep(
          position, span);
        motion_ticks, COMPETITION_ACCEL_TICKS);
    command = (COMPETITION_CRUISE_COUNTS * blend +
               (COMPETITION_SMOOTHSTEP_SCALE / 2U)) >> 10;
  }
  else if (s_continuous_mode != 0U)
  {
    command = COMPETITION_CRUISE_COUNTS;
  }
  else
  {
    OdometerControlProgress_t progress =
        Odometer_GetControlProgress();

    if ((progress.valid != 0U) &&
        (progress.lap_progress_tenths > s_max_lap_progress))
    {
      s_max_lap_progress = progress.lap_progress_tenths;
    }

    if ((progress.valid == 0U) ||
        (s_max_lap_progress <= COMPETITION_DECEL_START_PROGRESS))
    {
      command = COMPETITION_CRUISE_COUNTS;
    }
    else if (s_max_lap_progress >= COMPETITION_DECEL_END_PROGRESS)
    {
      command = COMPETITION_FINAL_COUNTS;
    }
    else
    {
      uint32_t span = COMPETITION_DECEL_END_PROGRESS -
                      COMPETITION_DECEL_START_PROGRESS;
      uint32_t position = s_max_lap_progress -
                          COMPETITION_DECEL_START_PROGRESS;
      uint32_t blend = CompetitionTasks_SmoothStep(
          position, span);
      uint32_t speed_drop = COMPETITION_CRUISE_COUNTS -
                            COMPETITION_FINAL_COUNTS;

      command = COMPETITION_CRUISE_COUNTS -
          ((speed_drop * blend +
            (COMPETITION_SMOOTHSTEP_SCALE / 2U)) >> 10);
    }
  }

  Car_SetLineFollowBaseCounts((int32_t)command);
}

说明：该代码依据运行时间和里程计圈程进度生成速度指令：起步阶段采用 SmoothStep 曲线平滑加速，接近终点时按标定进度平滑减速至最终速度，从而兼顾运行效率、循迹稳定性和停车精度。